# (Your Business Name Here) – Safe Work Procedure

# PORTABLE GRAIN AUGER

DO NOT use this machine unless you have been instructed

in its safe use and operation and have been given permission

## PERSONAL PROTECTIVE EQUIPMENT

## PRE-OPERATIONAL SAFETY CHECKS

Locate and ensure you are familiar with all machine operations and controls.

Ensure all guards are fitted, secure and functional. Do not operate if guards are missing or faulty.

Replace all safety signs that are worn, missing or illegible.

Inspect winch and cable (or other lifting system) for condition, security and operation.

If tractor operated, ensure the grain auger is attached according to manufacturer’s specification and confirm the PTO and all driveline components are in good condition and properly guarded.

## OPERATIONAL SAFETY CHECKS

Lower the auger to a horizontal position and lock before moving from one location to another.

When transporting, ensure the entire area above the auger and in the line of travel is clear of obstructions and overhead powerlines.

Slowly move the auger into working position on dry, level ground with the towing vehicle. Chock the wheels.

Securely support the auger by anchoring the intake end and/or supporting at the discharge end.

Only people authorised to operate the auger should be in the work area.

Keep clear of moving machine parts. Do not help the grain flow into the auger with your hands or feet.

Maintain a proper balance and secure footing. Keep the work area and machine clean and free of clutter.

Shut down the motor when refuelling (where self powered).

## ENDING OPERATIONS AND CLEANING UP

Remove any foreign material from in and around the machine parts.

Keep the work area and implement shed in a safe, clean and tidy condition.

## POTENTIAL HAZARDS AND INJURIES

Hair/clothing getting caught in moving machine parts.

Being struck by an uncontrolled spinning crank used to raise or lower the auger.

## DON’T

Do not use faulty equipment.  Report suspect machinery immediately.

Never wear loose clothing, jewellery, or have long hair untied while operating a grain auger.

Do not allow anyone on the tractor or grain auger when transporting.

Do not elevate an auger beyond its safe limits as it may become top-heavy when it is set up too high.

Do not attempt to clean out any blockages or jams until the machine is shut off and all moving parts come to a complete stop and the power source is isolated.

This safe work procedure does not necessarily cover all possible hazards associated with this equipment and should be used in conjunction with other references. 
It is designed as a guide to be used to compliment training and as a reminder to users prior to equipment use.

ALWAYS refer to the manufacturer’s instructions when writing your own safe work procedure.