# (Your Business Name Here) – Safe Work Procedure

# HORIZONTAL METAL BANDSAW

DO NOT use this machine unless you have been instructed

in its safe use and operation and have been given permission

## PERSONAL PROTECTIVE EQUIPMENT

| Safety glasses must be worn at all times in work areas. | Long and loose hair must be contained. | Hearing protection must be worn. |
| --- | --- | --- |
| Sturdy footwear must be worn at all times in work areas. | Close fitting/protective clothing must be worn. | Rings and jewellery must not be worn. |

## PRE-OPERATIONAL SAFETY CHECKS

Locate and ensure you are familiar with all machine operations and controls

Ensure all guards are fitted, secure and functional. Do not operate if guards are missing or faulty.

Check workspaces and walkways to ensure no slip/trip hazards are present.

Ensure the hydraulic damping mechanism functions.

Check that the blade is in good condition.

Ensure the blade’s speed, tension and tracking are properly adjusted.

Check coolant delivery system to allow for sufficient flow of coolant.

## OPERATIONAL SAFETY CHECKS

Lift the head of unit up and lock it in the upward position.

Set the angle of the vice or check it to ensure its squareness.

Clamp workpiece firmly into the vice. Long material must be supported.

Adjust blade guards to cover unused portion of blade.

Ensure hands are away from the blade, and then turn the machine on.

Allow the upper head assembly to come down slowly until the teeth are cutting the material.

Keep hands away from the blade and cutting area.

Turn off the machine and bring it to a complete standstill if the blade is to be lifted out of an uncompleted or jammed cut.

Stop the saw immediately if the blade develops a ‘click’. Report it to your supervisor.

Ensure the cutting head is locked in the upward position before removing workpiece from vice.

## ENDING OPERATIONS AND CLEANING UP

Switch off the machine when work completed.

Reset all guards to a fully closed position.

Before removing scrap pieces from the vice area or making adjustments, Stop the machine and bring it to a complete standstill.

Leave the machine in a safe, clean and tidy state.

## DON’T

Do not use faulty equipment. Immediately report suspect machinery.

Do not push down on the cutting head while it is cutting.

Never leave the machine running unattended.

This safe work procedure does not necessarily cover all possible hazards associated with this equipment and should be used in conjunction with other references. 
It is designed as a guide to be used to compliment training and as a reminder to users prior to equipment use.

ALWAYS refer to the manufacturer’s instructions when writing your own safe work procedure.