# (Your Business Name Here) – Safe Work Procedure

# HANDHELD EDGER – PETROL OPERATED

DO NOT use this machine unless you have been instructed

in its safe use and operation and have been given permission

## PERSONAL PROTECTIVE EQUIPMENT

## PRE-OPERATIONAL SAFETY CHECKS

Locate and ensure you are familiar with all machine operations and controls.

Check for loose/missing nuts, bolts and screws. Tighten and/or replace as needed.

Inspect fuel lines, tank, and area around carburetor for fuel leaks. Do not operate if leaks are found.

Ensure all guards are fitted, secure and functional. Do not operate if guards are missing or faulty.

Check that the cutting attachment is firmly attached and in safe operating condition.

Check that the front handle is adjusted for safe, comfortable operation.

## OPERATIONAL SAFETY CHECKS

Ensure no person or animal is endangered when operating equipment.

When starting:

- Lay the unit on a flat area and keep movable attachment parts clear of all obstacles.

- Firmly grasp right hand grip with left hand.

Before edging, plan your direction of travel so the unit will always be on your right side and so you walk on a hard surface whenever possible.

Hold the machine using both hands, with thumbs and fingers firmly encircling the handles.

Adjust the blade's depth of cut to produce a fine cut between edging and grass using a minimum blade depth, usually with about 13 mm of the blade into the ground.

When starting to edge, run the engine at full throttle, and lower the blade gently into the ground, being careful not to hit the concrete with the blade.

Blade depth may have to be adjusted due to differences in height between the hard surface and the top of the grass.

Shut down immediately if the unit starts to shake or vibrate.

Disconnect spark plug wire before you work on the unit or leave it unattended.

Allow the unit to cool before refuelling. Take care to avoid spilling fuel onto hot motor or exhaust.

## ENDING OPERATIONS AND CLEANING UP

After shutting down the engine, keep fingers and feet away from the cutting blade until all rotation stops.

Remove any foreign material from in and around the motor, blades and guards.

Keep the work area and implement shed in a safe, clean and tidy condition.

## POTENTIAL HAZARDS AND INJURIES

Moving components	.

Ejected material.

Vibration.

Noise.

Hair/clothing getting caught in moving machine parts.

Burns from exhaust.

## DON’T

Do not use faulty equipment.  Report suspect machinery immediately.

Do not work on slippery, uneven or unstable surfaces.

Do not adjust the blade height with engine running.

Do not force the blade. Move as quickly as the blade will allow.

This safe work procedure does not necessarily cover all possible hazards associated with this equipment and should be used in conjunction with other references. 
It is designed as a guide to be used to compliment training and as a reminder to users prior to equipment use.

ALWAYS refer to the manufacturer’s instructions when writing your own safe work procedure.